Pool Predictive Maintenance System

Performance Report & Model Analysis

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

Client: SPP System — Pool Maintenance Services

Region: Comunitat Valenciana, Spain

Regulatory Basis: Real Decreto 742/2013 · Decreto 85/2018

Date: July 01, 2026

Fleet Size: 486 pools

Data Period: April 2017 – December 2022 (5.7 years)

# Table of Contents

1.  Executive Summary

2.  Regulatory Context

3.  How the System Works

4.  Why This Approach

5.  Model Comparison (V3 vs V5 Candidates)

6.  Limitations & Known Risks

7.  Example Outputs

8.  Recommended Next Steps

# 1. Executive Summary

This system predicts when each swimming pool in your fleet needs its next maintenance visit and what chemicals the technician should bring — reducing unnecessary trips while ensuring no pool falls below the safety standards set by Spanish regulation.

Built on 5.7 years of real maintenance data from 486 pools serviced by SPP System in Alicante, the system delivers three key capabilities:

Visit Timing: Recommends when to visit each pool (accurate to within ~1 day on average), flagging pools that need attention sooner than the standard schedule.

Water Chemistry Forecasting: Predicts pH, chlorine, and turbidity levels at the next visit, with pH predictions accurate to within 0.08 pH units — matching the precision of handheld meters.

Chemical Demand: Calculates exactly how many kilograms of sodium hypochlorite, pH corrector, or flocculant the technician should bring, based on the predicted water chemistry and pool volume.

The system is grounded in the thresholds defined by Real Decreto 742/2013 and applies conservative safety logic: when in doubt, it recommends visiting earlier rather than later.

# 2. Regulatory Context

All predictions and prescriptions are anchored to the water quality standards set by Spanish national and Valencian regional regulation:

| Parameter | Compliant Range | Consequence of Breach |
| --- | --- | --- |
| Free Chlorine | 0.5 – 2.0 mg/L (ideal) 0.5 – 5.0 mg/L (safe) | < 0.5: Pathogen risk > 5.0: Mandatory pool closure |
| pH | 7.2 – 8.0 | Skin/eye irritation; reduced disinfection efficacy |
| Turbidity | ≤ 5 NTU | Reduced visibility; potential contamination |
| Monitoring | Daily autocontrol required | Decreto 85/2018 (Comunitat Valenciana) |

Important note on chlorine levels: 65% of all readings in the dataset show chlorine above 2.0 mg/L. This is standard practice in Spanish pool maintenance — technicians intentionally overdose chlorine because it degrades rapidly in Mediterranean sun. Chlorine between 2.0 and 5.0 mg/L is common, safe, and not flagged as a problem by this system. Only levels below 0.5 mg/L (pathogen risk) or above 5.0 mg/L (chemical burn / mandatory closure) trigger safety alerts.

# 3. How the System Works

The system processes each pool's maintenance history — readings, chemical applications, and equipment settings — to produce three outputs for each pool:

### Visit Timing

Rather than predicting the exact number of days until the next visit, the system first recognizes the strong seasonal pattern in your fleet: pools are visited every ~2 days in summer and every ~7 days in winter. It then predicts which pools need to deviate from this schedule. A pool with dropping chlorine or drifting pH will be flagged for an earlier visit; a stable pool can safely wait longer.

### Water Chemistry Forecasting

Three separate models predict what the pH, chlorine, and turbidity will be at the next visit. These predictions are based on the pool's recent history (last 2–3 readings), the rate of change, current chemical applications, and pool characteristics (volume, type, equipment). The predictions drive the chemical prescription below.

### Chemical Prescription

Based on the predicted water chemistry and the pool's volume, the system calculates the exact kilograms of sodium hypochlorite (chlorine), sodium bisulfate (pH reducer), or flocculant needed to bring the water back to ideal levels. This is done using standard mass-balance chemistry equations grounded in the regulatory thresholds.

### System Flow

Data Flow: Technician readings → Feature engineering (trends, headroom, breach history) → ML models (XGBoost) → Visit recommendation + Chemical prescription

# 4. Why This Approach

### Seasonal Deviation Framing

Your technicians already follow a sensible seasonal schedule. The model doesn't try to replace it — instead, it identifies which pools should deviate from the standard schedule. This is more useful than a raw "visit in X days" prediction because the seasonal pattern already accounts for ~85% of the variation. The model focuses on the remaining signal: which specific pools are riskier than average right now.

### Safety-First Chlorine Definition

The system defines a "chlorine breach" as readings below 0.5 mg/L or above 5.0 mg/L — not the narrower 0.5–2.0 mg/L "ideal" range. This was a deliberate choice because 65% of readings fall in the 2.0–5.0 range, which is standard practice in your fleet. Using the strict definition would flag two-thirds of all readings as problematic, overwhelming the system with false alarms. The broader safety definition correctly identifies only genuinely dangerous conditions (0.6% of readings).

### Temporal Train/Test Split

The model is evaluated using a strict time-based split: it trains on data before April 2022 and is tested on data from April–December 2022. This ensures the model never "sees the future" during training, giving honest performance estimates. The metrics in this report reflect how the model performs on genuinely unseen future data.

### Safety-Weighted Training

During training, data points where a safety breach occurred at the next visit are given 3× more weight. This biases the model toward recommending earlier visits when conditions look risky — a conservative, safety-first approach appropriate for a public health application.

### Static Data Backfilling

Pool physical characteristics (volume, surface area, filter specs) were originally recorded on only ~1.4% of rows. The pipeline propagates each pool's known values across all its rows, and fills pools with no data at all using the fleet median. This enables volume-normalized chemical prescriptions for every pool, though prescriptions for the 28% of pools using imputed volumes should be treated as approximate.

# 5. Model Comparison

We evaluated six different machine learning approaches to determine the best model for each prediction target. All models were trained on the same data with the same features and tested on the same held-out period (April–December 2022).

## Visit Timing

Predicts when each pool should next be visited. The mean absolute error (MAE) represents the average prediction error in days.

| Model | MAE (days) | RMSE | R² |  |
| --- | --- | --- | --- | --- |
| LightGBM | 0.937 | 1.659 | 0.552 | ✓ Best |
| XGBoost | 0.936 | 1.659 | 0.552 |  |
| Gradient Boosting | 0.946 | 1.665 | 0.549 |  |
| Random Forest | 0.946 | 1.674 | 0.544 |  |
| CatBoost | 0.973 | 1.688 | 0.536 |  |
| Extra Trees | 0.953 | 1.692 | 0.534 |  |

The visit timing model is accurate to within approximately 1 day on average. LightGBM and XGBoost perform virtually identically (difference of 0.001 days). The R² of 0.55 means the model explains about 55% of the variance in visit scheduling — the remaining variance comes from operational factors (crew availability, route optimization) that the model correctly does not try to predict.

## pH Prediction

| Model | MAE (pH) | RMSE | R² |  |
| --- | --- | --- | --- | --- |
| Extra Trees | 0.082 | 0.117 | 0.530 | ✓ Best |
| LightGBM | 0.082 | 0.118 | 0.528 |  |
| Gradient Boosting | 0.082 | 0.118 | 0.527 |  |
| XGBoost | 0.082 | 0.118 | 0.526 |  |
| Random Forest | 0.082 | 0.118 | 0.524 |  |
| CatBoost | 0.084 | 0.119 | 0.522 |  |

The pH model is accurate to within 0.082 pH units — this is within the measurement precision of standard handheld pH meters (±0.1 pH). In practical terms, the model's forecast is as accurate as the instrument itself in most scenarios.

## Chlorine Prediction

| Model | MAE (mg/L) | RMSE | R² |  |
| --- | --- | --- | --- | --- |
| LightGBM | 0.529 | 0.732 | 0.277 | ✓ Best |
| XGBoost | 0.529 | 0.733 | 0.276 |  |
| Gradient Boosting | 0.529 | 0.733 | 0.276 |  |
| Extra Trees | 0.529 | 0.735 | 0.273 |  |
| Random Forest | 0.530 | 0.735 | 0.272 |  |
| CatBoost | 0.532 | 0.735 | 0.272 |  |

The chlorine model is accurate to within 0.53 mg/L. Given the wide safe operating range (0.5–5.0 mg/L), this level of accuracy is sufficient for reliable safety warnings and chemical dosage calculations. The lower R² (0.28) reflects the inherent unpredictability of chlorine levels — they are heavily influenced by UV exposure, bather load, and temperature, none of which are captured in this dataset.

## Turbidity Prediction

| Model | MAE (NTU) | RMSE | R² |  |
| --- | --- | --- | --- | --- |
| CatBoost | 0.097 | 0.176 | 0.688 | ✓ Best |
| Extra Trees | 0.098 | 0.176 | 0.687 |  |
| LightGBM | 0.098 | 0.180 | 0.675 |  |
| XGBoost | 0.098 | 0.180 | 0.675 |  |
| Gradient Boosting | 0.099 | 0.181 | 0.670 |  |
| Random Forest | 0.101 | 0.189 | 0.643 |  |

The turbidity model has the highest accuracy of all targets (R² = 0.69, MAE = 0.10 NTU). Given the regulatory limit of 5 NTU, an error of 0.10 NTU is negligible. Turbidity breaches are extremely rare in this fleet (0.01% of readings), indicating consistently clean water.

## Summary: Does Any Model Beat XGBoost?

No model significantly outperforms XGBoost. The best model for each target offers at most a marginal (<1%) improvement. This is typical for structured tabular data, where modern gradient boosting methods converge to very similar performance. The current XGBoost-based production pipeline is well justified — it offers the advantage of native model serialization (JSON), mature SHAP explainability, and the widest ecosystem support for deployment.

# 6. Limitations & Known Risks

We identified the following limitations through independent analysis of the data and pipeline. These are presented constructively — most can be addressed with additional data collection or incremental system improvements.

### Pool Physical Data is Partially Imputed

Pool volume was originally recorded for only 349 of 486 pools (72%). The remaining 137 pools use the fleet median (220.0 m³). Chemical dosage prescriptions for these pools should be treated as approximate. For motor_count, 60% of pools have imputed values.

Recommendation: A one-time data collection exercise to record volume and surface area for all pools would significantly improve dosage accuracy.

### Some Pools Have Very Limited History

The median pool has 455 readings, but 13 pools have fewer than 5 readings — too few for the lag and rolling features to be meaningful. For these pools, predictions rely heavily on fleet-wide patterns rather than pool-specific history. Predictions for pools with fewer than ~10 historical readings should be treated as lower-confidence estimates.

### Safety Alert Precision is Limited by Data Imbalance

Only 0.6% of readings have dangerously low chlorine (< 0.5 mg/L) — an imbalance ratio of approximately 1:169. The safety alert classifier catches ~80% of true low-chlorine events (recall = 0.80), but ~97% of its alerts are false positives (precision = 0.03). This is a deliberate tradeoff: the system prioritizes not missing a genuine safety event, at the cost of some unnecessary alerts. As more data accumulates (especially positive breach examples), precision will improve.

### Winter Predictions May Be Less Reliable

The dataset contains 3.1× more summer readings than winter readings (60.1% vs 19.2%). The test set is dominated by summer months (April–September). Winter-specific patterns have less training data, and winter predictions have not been as extensively validated.

### External Factors Not Captured

The system does not currently account for: weather conditions (temperature, rainfall, UV index), bather load (pool usage patterns), algae events beyond turbidity measurements, chemical brand/quality variability, technician measurement variability, or equipment failures. Adding weather data (available via public APIs) would likely improve chlorine predictions the most, as UV exposure is the primary driver of chlorine degradation.

### Fleet-Specific Model

This model is trained on data from a single service company (32 technicians) in one region (Comunitat Valenciana). The seasonal baselines, chlorine dosing practices, and chemical product types are specific to this fleet and Mediterranean climate. Deploying this model for a different region (e.g., northern Spain, France, Germany) would require retraining on local data, as seasonal patterns, regulations, and maintenance practices differ significantly.

# 7. Example Outputs

Below are three representative prescriptions generated by the system, showing the range of recommendations from routine maintenance to urgent intervention.

### Example 1: Stable Pool — Routine Visit

| Pool | Bulevar Doce (1013) |
| --- | --- |
| Current pH | 7.6 |
| Current Chlorine | 3.1 mg/L |
| Current Turbidity | 0.3 NTU |
| Predicted pH (next visit) | 7.57 |
| Predicted Chlorine | 3.09 mg/L |
| Recommended Visit | 6 days — Routine |
| Chemical Prescription | No chemicals needed |

### Example 2: pH Drift — Early Visit Recommended

| Pool | Residencial Alkabir (426) |
| --- | --- |
| Current pH | 7.2 (at lower limit) |
| Current Chlorine | 5.0 mg/L |
| Current Turbidity | 1.1 NTU |
| Predicted pH (next visit) | 7.25 |
| Predicted Chlorine | 3.03 mg/L |
| Recommended Visit | 5 days — Soon |
| Chemical Prescription | Monitor pH closely; headroom to limit is 0.00 |

### Example 3: Safety Alert — Immediate Action

| Pool | Hercules II Bloque II (9914) |
| --- | --- |
| Current pH | 7.4 |
| Current Chlorine | 0.8 mg/L |
| Current Turbidity | 1.2 NTU |
| Predicted pH (next visit) | 7.39 |
| Predicted Chlorine | 2.34 mg/L |
| Safety Alert | 49.3% probability of chlorine dropping below 0.5 mg/L |
| Recommended Visit | 6 days — Immediate |
| Chemical Prescription | Bring sodium hypochlorite 15%; monitor chlorine trend |

# 8. Recommended Next Steps

To move from prototype to production, we recommend the following phases:

### Phase 1: Data Quality (1–2 months)

Collect pool volume and surface area for all pools currently using fleet-median estimates.

Standardize data entry to reduce measurement variability across technicians.

Integrate weather data (temperature, UV index) from public meteorological APIs — this is expected to improve chlorine predictions the most.

### Phase 2: Deployment (2–3 months)

Deploy the model as a cloud-hosted API that receives each day's readings and returns visit recommendations and prescriptions.

Build a technician-facing mobile app or integrate with existing route-planning software.

Set up automated retraining (monthly) as new data accumulates.

Implement monitoring dashboards to track prediction accuracy over time.

### Phase 3: Expansion (3–6 months)

Extend the model to new regions — requires 6+ months of local data for retraining.

Add microbiological monitoring integration (monthly lab tests per RD 742/2013).

Explore anomaly detection for equipment failures and unusual water chemistry events.

Integrate bather load data (if available) to improve chlorine demand forecasting.

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

Report generated July 01, 2026